AeroGrid Turbine Health Report

To: Chief Technology Officer, AeroGrid

From: Abhisheksinh Vanrajsinh Rathod, Data Engineering

Date: June 2026

Subject: Urgent maintenance for two turbines and a proposed monitoring pipeline

Findings

I analysed 5,000 telemetry readings across the 10-turbine fleet, and two units need immediate inspection.

T-04 is running hot — it averages 90.6°C against the 85°C limit, roughly 25°C above the rest of the fleet. That usually points to bearing friction or a failing cooling system, and sustained heat will keep degrading its seals and bearings until it fails.

T-07 is vibrating too much — peaking at 25.0 mm/s against the 15 mm/s limit, and consistently rather than as a single spike. That pattern suggests blade imbalance or rotor misalignment, which fatigues the metal and risks structural cracking.

The remaining eight turbines are operating normally.

Architecture justification

The core problem is that the current server can’t keep up with the constant flow of data, so warning signs slip through unnoticed. I’d route the telemetry into a message queue (Apache Kafka or AWS Kinesis) that absorbs the high-volume stream and separates collection from processing, so nothing is lost under load. A serverless function (AWS Lambda) then runs the anomaly checks live and fires an instant alert (Amazon SNS) the moment a turbine crosses a limit, so the team can act before a failure rather than after one. The data then splits into hot storage — a time-series database powering live dashboards — and cold storage, an S3 data lake for cheap long-term archiving.

Cost optimisation

To keep costs down, I’d add S3 lifecycle rules that automatically move older readings into cheaper storage tiers while keeping recent data fast to access. Paired with Lambda’s pay-per-use pricing, where there’s no charge for idle servers, the pipeline stays affordable even as the data keeps growing.

My recommendation: schedule maintenance for T-04 and T-07 now, and stand up this pipeline so the next early warning is caught in time.